IoT Smart Safety Helmet

for Coal Miners

Using ESP32

Complete Beginner's Guide

From Zero to Working Project

| ✅ Components to Buy | ✅ Simulation in Wokwi |
| --- | --- |
| ✅ Hardware Assembly | ✅ Full ESP32 Code |
| ✅ Live IoT Dashboard + Testing & Troubleshooting |  |

A Confluence Project

# CHAPTER 1: Introduction & Project Overview

### What Are We Building?

This project is a Smart Safety Helmet for coal miners that uses sensors to monitor dangerous conditions underground and alerts miners and supervisors in real-time over the internet. Think of it as a helmet that has a 'brain' (ESP32) which reads the environment and shouts for help when something dangerous happens.

### Why Is This Important?

- Coal mines have dangerous gases like Carbon Monoxide (CO) and Methane that are invisible and odorless — miners cannot detect them without instruments

- Temperature and air pressure can become deadly deep underground

- A slip or fall inside a mine can trap a worker unconscious with no way to call for help

- GPS tracking helps rescuers find miners instantly in emergencies

- Traditional helmets are just plastic — this project makes them SMART

### Why ESP32 Instead of Arduino?

The original project used Arduino Uno + ESP8266. We are upgrading to ESP32 for good reasons:

| Feature | Arduino Uno + ESP8266 | ESP32 (Our Choice) |
| --- | --- | --- |
| WiFi | Separate ESP8266 chip needed | Built-in WiFi |
| Bluetooth | Not available | Built-in Bluetooth (BLE) |
| CPU Speed | 16 MHz | 240 MHz (15x faster) |
| Analog Pins | 6 pins | 18 pins (more sensors!) |
| GPIO Pins | 14 digital | 34 GPIO pins |
| I2C / SPI | 1 each | Multiple I2C & SPI buses |
| Cost | Arduino ₹300 + ESP8266 ₹150 | ESP32 board ~₹350 (all-in-one) |
| Size | Two boards needed | One compact board |

💡 TIP: The ESP32 does the job of Arduino + ESP8266 in one single chip. That means fewer wires, less power consumption, and it fits better inside a helmet!

### Complete System Overview

Here is what the complete system looks like at a high level:

INPUT (Sensors on Helmet)

MQ-2  →  Detects LPG / Smoke / CO

MQ-7  →  Detects Carbon Monoxide (CO) specifically

MQ-135  →  Detects Air Quality / NH3 / Benzene

KE-25  →  Measures Oxygen (O2) level

DHT-22  →  Measures Temperature + Humidity

BMP-280  →  Measures Air Pressure + Altitude

MPU-6050  →  Detects Falls (Accelerometer + Gyroscope)

SW-420  →  Detects Vibration / Mine Collapse

NEO-6M GPS  →  Tracks Location of Miner

Push Button  →  SOS Emergency Alert

PROCESSING (ESP32 Brain)

Reads all sensor data every 2 seconds

Checks against safe threshold limits

Triggers buzzer alarm if danger detected

Sends data to cloud (Blynk / ThingSpeak) via WiFi

Sends SMS alert via Twilio if SOS pressed or critical danger

OUTPUT (Dashboard + Alerts)

Blynk App on phone: Live sensor readings with color indicators

ThingSpeak: Graphs and data logging for supervisors

Buzzer: Loud alarm on the helmet itself

Email/SMS: Automatic alerts when thresholds exceeded

# CHAPTER 2: Components List

This chapter lists every single component you need to buy, what it does, and why we chose that specific model. Do NOT buy cheaper replacements for safety-critical sensors.

## 2.1  Microcontroller — The Brain

| Component | Model | Approx. Cost | Why This One? |
| --- | --- | --- | --- |
| ESP32 Dev Board | ESP32-WROOM-32 / NodeMCU ESP32 | ₹350–450 | All-in-one: WiFi + Bluetooth + 34 GPIO + dual core + lots of analog pins. Runs our full code without needing any extra chips. |

💡 TIP: Buy the 38-pin ESP32 development board. It has all pins already on headers. Works directly with breadboard. Look for 'ESP32 WROOM DevKit V1' on Amazon/Robocraze.

## 2.2  Gas Detection Sensors — The Lungs

| Component | Model | Approx. Cost | Why This One? |
| --- | --- | --- | --- |
| Combustible Gas Sensor | MQ-2 | ₹80–120 | Detects LPG, Methane, Propane, Butane, Smoke, Hydrogen. The most common gas sensor for beginners. Detects explosions before they happen. |
| Carbon Monoxide Sensor | MQ-7 | ₹120–160 | CO is the #1 killer in mines — called 'the silent killer'. MQ-7 is specifically tuned for CO with high accuracy. Works in LOW (1.5V) + HIGH (5V) duty cycle heating. |
| Air Quality Sensor | MQ-135 | ₹80–120 | Detects Ammonia (NH3), Benzene, CO2, Alcohol vapors. Measures overall air quality index. Essential for checking breathability. |

⚠️ WARNING: These MQ sensors need a 24-48 hour 'burn-in' period when first powered on. Connect them to 5V for 24 hours before using them for measurements. This stabilizes the internal heating element.

## 2.3  Oxygen Sensor — Life Support Monitor

| Component | Model | Approx. Cost | Why This One? |
| --- | --- | --- | --- |
| Electrochemical O2 Sensor | KE-25 (Grove/DFRobot) | ₹800–1500 | Normal air has 20.9% O2. Below 19.5% causes dizziness. Below 16% causes loss of consciousness. Below 6% causes death. KE-25 is an electrochemical cell — far more accurate than cheap semiconductor sensors for O2 measurement. |

📝 NOTE: The KE-25 produces a small analog voltage proportional to O2 concentration. You will need a simple op-amp amplifier circuit (LM358) or buy the DFRobot SEN0340 breakout board which already includes amplification.

## 2.4  Environmental Sensors — Climate Monitor

| Component | Model | Approx. Cost | Why This One? |
| --- | --- | --- | --- |
| Temp + Humidity Sensor | DHT-22 (AM2302) | ₹150–200 | More accurate than DHT-11. Range: -40 to +80°C, 0–100% RH. In your notes you mentioned DHT-11 or DHT-22 — we recommend DHT-22 for better accuracy in extreme mine conditions. |
| Barometric Pressure Sensor | BMP-280 | ₹120–180 | Measures air pressure AND altitude. In a collapsing mine, pressure changes rapidly. Also detects how deep underground the miner is. Uses I2C protocol — easy to connect. |

## 2.5  Motion & Fall Detection — Safety Guardian

| Component | Model | Approx. Cost | Why This One? |
| --- | --- | --- | --- |
| IMU Accelerometer/Gyroscope | MPU-6050 | ₹80–120 | 6-axis sensor: 3-axis accelerometer + 3-axis gyroscope. When a miner falls, the sudden change in acceleration is detected. The gyroscope detects if the helmet is tilted beyond a safe angle (e.g., lying flat). Uses I2C — same bus as BMP-280. |
| Vibration Sensor | SW-420 | ₹30–60 | Detects mine wall vibrations or explosions. The SW-420 is a simple tilt/shock sensor with adjustable sensitivity via potentiometer. When vibration is detected, it sends a digital HIGH signal to ESP32. |

## 2.6  GPS Tracking — Location System

| Component | Model | Approx. Cost | Why This One? |
| --- | --- | --- | --- |
| GPS Module | NEO-6M with Antenna | ₹350–500 | Provides latitude/longitude of the miner. When sent to the dashboard, supervisors can see where each miner is on a map. NEO-6M is reliable, widely available, and has many libraries. Note: GPS works best near mine entrances — deep underground GPS signals weaken. |

💡 TIP: The NEO-6M comes with a small ceramic patch antenna. For better signal inside mine tunnels, you can upgrade to an active external antenna with a longer cable and mount it near the mine entrance.

## 2.7  Output & Communication Components

| Component | Model | Approx. Cost | Why This One? |
| --- | --- | --- | --- |
| Active Buzzer | 5V Active Piezo Buzzer | ₹20–40 | Produces loud alarm sound when danger detected. 'Active' means it makes sound when you just apply 5V — no extra programming needed for the sound frequency. |
| Push Button | Momentary Push Button (NO type) | ₹10–20 | The SOS emergency button. When miner presses it, an immediate alert is sent to the supervisor's dashboard and phone. Use Normally Open (NO) type. |
| OLED Display | 0.96 inch SSD1306 OLED (I2C) | ₹120–180 | Shows live sensor readings ON the helmet itself. The miner can read CO level, temperature, etc. without needing a phone. Uses same I2C bus. |
| 16x2 LCD (Optional) | LCD 1602 with I2C module | ₹80–120 | Alternative to OLED if OLED is not available. The I2C module reduces connections to just 4 wires. |

## 2.8  Power & Miscellaneous

| Component | Model | Approx. Cost | Why This One? |
| --- | --- | --- | --- |
| Li-Po Battery | 3.7V 2000mAh Li-Po | ₹250–400 | Rechargeable. Compact. Fits in helmet. 2000mAh gives ~4-6 hours of operation. Rechargeable via micro-USB on the ESP32 board. |
| TP4056 Charger Module | TP4056 with protection | ₹25–40 | Safely charges the Li-Po battery. Has overcharge and over-discharge protection — critical for safety. |
| Breadboard + Jumper Wires | 830 tie-point breadboard + 65pc wires | ₹150–200 | For prototyping before final PCB. Male-to-male and male-to-female sets needed. |
| 10kΩ Resistors | 10kΩ (pack of 20) | ₹20 | Pull-up resistors for DHT-22, push button, and other digital sensors. |
| 4.7kΩ Resistors | 4.7kΩ (pack of 20) | ₹20 | I2C pull-up resistors (if your modules don't have them built-in). |

## 2.9  Complete Shopping List Summary

| # | Component | Model | Qty | Price (₹) |
| --- | --- | --- | --- | --- |
| 1 | ESP32 Dev Board | WROOM-32 38-pin | 1 | ₹400 |
| 2 | Gas Sensor | MQ-2 | 1 | ₹100 |
| 3 | CO Sensor | MQ-7 | 1 | ₹140 |
| 4 | Air Quality Sensor | MQ-135 | 1 | ₹100 |
| 5 | Oxygen Sensor | KE-25 / DFR SEN0340 | 1 | ₹1200 |
| 6 | Temp+Humidity | DHT-22 | 1 | ₹180 |
| 7 | Pressure Sensor | BMP-280 | 1 | ₹150 |
| 8 | IMU Sensor | MPU-6050 | 1 | ₹100 |
| 9 | Vibration Sensor | SW-420 | 1 | ₹45 |
| 10 | GPS Module | NEO-6M + Antenna | 1 | ₹450 |
| 11 | Buzzer | Active 5V Piezo | 1 | ₹30 |
| 12 | OLED Display | SSD1306 0.96" I2C | 1 | ₹150 |
| 13 | Push Button | Momentary NO | 2 | ₹20 |
| 14 | Li-Po Battery | 3.7V 2000mAh | 1 | ₹350 |
| 15 | TP4056 Module | With protection | 1 | ₹35 |
| 16 | Breadboard | 830 tie-point | 1 | ₹80 |
| 17 | Jumper Wires | 65pc set | 1 | ₹120 |
| 18 | Resistors | 10kΩ + 4.7kΩ packs | 2 | ₹40 |
| ESTIMATED TOTAL |  |  |  | ~₹3,690 |

💡 TIP: Order from Robocraze, Robu.in, or Amazon India. Search for exact model numbers. Avoid very cheap (no-brand) gas sensors — they can give wrong readings in real emergencies!

# CHAPTER 3: Simulation with Wokwi

Before touching real hardware, we SIMULATE the circuit on a computer. This lets you test your code without risk of burning anything. Wokwi is a free, browser-based Arduino/ESP32 simulator — no installation needed.

### Step 1: Open Wokwi

- Open your browser and go to: https://wokwi.com

- Click on 'New Project'

- Select 'ESP32' from the microcontroller list

- You will see an ESP32 chip on screen with a code editor on the right

### Step 2: Add Components in Wokwi

Click the blue '+' button (Add Part) to add each component. Search by name:

- Search 'DHT22' → Add it → Click on it → Set pin connections

- Search 'MPU6050' → Add I2C sensor

- Search 'BMP280' → Add pressure sensor

- Search 'Gas sensor' → Add MQ-2 type (or use potentiometer to simulate analog reading)

- Search 'Buzzer' → Add active buzzer

- Search 'LED' → Add 3 LEDs (Green=Safe, Yellow=Warning, Red=Danger)

- Search 'Button' → Add push button for SOS

- Search 'SSD1306' → Add OLED display

### Step 3: Wiring in Simulator

Click on a component pin and drag to ESP32 pin to connect. Follow this wiring:

| Sensor / Component | ESP32 Pin | Notes |
| --- | --- | --- |
| DHT-22 DATA | GPIO 4 | Also connect 10kΩ pull-up between DATA and 3.3V |
| DHT-22 VCC | 3.3V |  |
| DHT-22 GND | GND |  |
| MQ-2 AOUT | GPIO 34 (ADC) | Analog output - ADC1 channel |
| MQ-2 DOUT | GPIO 35 (ADC) | Digital threshold output |
| MQ-2 VCC | 5V | Needs 5V for heater |
| MQ-7 AOUT | GPIO 32 (ADC) |  |
| MQ-7 VCC | 5V |  |
| MQ-135 AOUT | GPIO 33 (ADC) |  |
| MQ-135 VCC | 5V |  |
| KE-25 / O2 Sensor | GPIO 36 (ADC) | Analog voltage output |
| BMP-280 SDA | GPIO 21 | I2C Data - shared bus |
| BMP-280 SCL | GPIO 22 | I2C Clock - shared bus |
| BMP-280 VCC | 3.3V |  |
| MPU-6050 SDA | GPIO 21 | Same I2C bus as BMP280 |
| MPU-6050 SCL | GPIO 22 | Same I2C bus as BMP280 |
| MPU-6050 VCC | 3.3V |  |
| MPU-6050 INT | GPIO 19 | Interrupt pin for motion detect |
| OLED SDA | GPIO 21 | Same I2C bus |
| OLED SCL | GPIO 22 | Same I2C bus |
| OLED VCC | 3.3V |  |
| SW-420 OUT | GPIO 18 | Digital output |
| SW-420 VCC | 3.3V |  |
| NEO-6M TX | GPIO 16 (RX2) | UART2 receive |
| NEO-6M RX | GPIO 17 (TX2) | UART2 transmit |
| NEO-6M VCC | 3.3V |  |
| Buzzer + | GPIO 26 | Active buzzer positive |
| Buzzer - | GND |  |
| SOS Button | GPIO 5 | Other end to GND, use INPUT_PULLUP in code |

📝 NOTE: In Wokwi, you can simulate analog sensor values by connecting a potentiometer to the ADC pins instead of actual MQ sensors. Rotate the pot knob to simulate 'increasing gas levels' and watch the alarm trigger.

### Step 4: Simulate and Test

- Paste the code (from Chapter 5) into the Wokwi editor

- Click the green Play button (▶) to start simulation

- The serial monitor at the bottom shows sensor readings

- Twist the potentiometer past the threshold → Buzzer should activate

- Press the SOS button → See SOS alert in serial monitor

- Test each sensor scenario before moving to real hardware

💡 TIP: Wokwi link to start: wokwi.com/projects — You can also share your simulation with a URL. Show this to your Confluence judges as proof of testing!

# CHAPTER 4: Hardware Assembly

Now we build the real circuit! Follow these steps carefully. Work on a clean, flat surface. Never connect power while wiring — always disconnect the battery first.

## 4.1  Safety First!

⚠️ WARNING: Always disconnect the battery or USB before changing any wires. MQ sensors get HOT during warm-up — do not touch the metal mesh part. Do not short-circuit the Li-Po battery — it can catch fire.

## 4.2  Tools You Need

- Breadboard (830 tie-points)

- Jumper wires (male-to-male and male-to-female)

- USB cable (micro-USB for ESP32 programming)

- Laptop/PC with Arduino IDE installed

- Multimeter (optional but helpful for testing connections)

- Small screwdriver

## 4.3  Step-by-Step Assembly

### Phase 1: Mount the ESP32

- Place the ESP32 development board in the CENTER of the breadboard, straddling the center groove

- The ESP32's pins should go into the breadboard holes — it holds itself

- The USB port should face the edge of the breadboard for easy access

- Connect a red jumper wire from ESP32 3.3V pin to the breadboard's positive (+) rail

- Connect a black wire from ESP32 GND pin to the breadboard's negative (-) rail

💡 TIP: The ESP32 has BOTH 3.3V and 5V (Vin) output when powered via USB. 3.3V goes to sensitive digital sensors. 5V goes to gas sensors that need more power for their heating elements.

### Phase 2: Connect I2C Sensors (BMP280, MPU6050, OLED)

I2C (Inter-Integrated Circuit) is a communication protocol that allows multiple sensors to share just 2 wires (SDA and SCL). This is like a telephone party line — everyone shares the same wire, but each sensor has a unique address.

- Connect a wire from ESP32 GPIO 21 to a row on the breadboard — this is your SDA bus line

- Connect a wire from ESP32 GPIO 22 to another row — this is your SCL bus line

- Connect BMP280: SDA pin → SDA row, SCL pin → SCL row, VCC → 3.3V rail, GND → GND rail

- Connect MPU6050: SDA pin → SDA row, SCL pin → SCL row, VCC → 3.3V rail, GND → GND rail, INT → GPIO 19

- Connect OLED: SDA → SDA row, SCL → SCL row, VCC → 3.3V rail, GND → GND rail

- Add 4.7kΩ pull-up resistors: one between SDA and 3.3V, one between SCL and 3.3V

📝 NOTE: I2C Addresses: BMP280 = 0x76 or 0x77, MPU6050 = 0x68 or 0x69, OLED = 0x3C. If two sensors have the same address, change the address jumper on one of them (look for an ADR solder pad).

### Phase 3: Connect Gas Sensors (MQ-2, MQ-7, MQ-135)

- MQ sensors have 4 pins: VCC, GND, AOUT (analog), DOUT (digital)

- Connect all MQ sensor VCC pins to the 5V (Vin) rail — NOT 3.3V

- Connect all GND pins to the GND rail

- MQ-2 AOUT → GPIO 34 (use a jumper wire)

- MQ-7 AOUT → GPIO 32

- MQ-135 AOUT → GPIO 33

- Connect a small LED (with 220Ω resistor) to each DOUT pin to visually see when threshold is exceeded

⚠️ WARNING: ESP32 ADC pins work at 3.3V maximum. MQ sensors output 0–5V. You MUST use a voltage divider (two resistors: 10kΩ and 10kΩ) between the AOUT and the ESP32 ADC pin to bring voltage down from 5V to 2.5V. Without this, you can permanently damage the ESP32!

Voltage Divider: MQ AOUT → 10kΩ → ESP32 ADC Pin → 10kΩ → GND

### Phase 4: Connect DHT-22

- DHT-22 has 3 pins (on module): VCC, DATA, GND

- VCC → 3.3V rail

- GND → GND rail

- DATA → GPIO 4

- Add a 10kΩ resistor between DATA pin and VCC (3.3V) — this is called a pull-up resistor

💡 TIP: The DHT-22 module usually already has the pull-up resistor built-in. If you bought the bare sensor (4 pins), you must add the resistor yourself.

### Phase 5: Connect MPU-6050 (Already done in Phase 2) & SW-420

- SW-420 has 3 pins: VCC, GND, OUT

- VCC → 3.3V, GND → GND, OUT → GPIO 18

- The small blue potentiometer on the SW-420 adjusts sensitivity — turn it clockwise for more sensitivity

### Phase 6: Connect NEO-6M GPS

- GPS module has 4 pins: VCC, GND, TX, RX

- VCC → 3.3V (NEO-6M works on 3.3V)

- GND → GND

- GPS TX → ESP32 GPIO 16 (this is ESP32's RX2 — it RECEIVES what GPS sends)

- GPS RX → ESP32 GPIO 17 (ESP32 TX2 — sends commands TO GPS)

- Attach the small external GPS antenna by clicking it onto the U.FL connector on the module

💡 TIP: GPS takes 30-60 seconds to get its first 'fix' (find satellites). The blue LED on the NEO-6M blinks once per second when it has a fix. Test this near a window or outdoors first.

### Phase 7: Connect Buzzer and SOS Button

- Active Buzzer: Positive (+) pin → GPIO 26, Negative (-) → GND

- SOS Button: One leg → GPIO 5, Other leg → GND

- In code, we use INPUT_PULLUP mode, so no external resistor needed for the button

### Phase 8: Connect KE-25 Oxygen Sensor

- The KE-25 raw sensor needs amplification. If you have the DFRobot SEN0340 breakout board:

- VCC → 3.3V, GND → GND, AOUT → GPIO 36

- If using raw KE-25: Connect through LM358 op-amp circuit (ask your guide/teacher for help with this circuit)

## 4.4  Final Check Before Powering On

- All 5V sensors: MQ-2, MQ-7, MQ-135 are connected to Vin (5V), NOT 3.3V

- All I2C sensors (BMP280, MPU6050, OLED) are on 3.3V

- Voltage dividers are in place for all MQ sensor AOUT connections

- Pull-up resistor on DHT-22 data line

- No bare wires touching each other

- GPS antenna is attached

- Button connected correctly (one side to pin, other to GND)

⚠️ WARNING: DOUBLE-CHECK all power connections before connecting USB! Use a multimeter to check: 3.3V rail shows ~3.3V, 5V (Vin) rail shows ~5V, GND rail shows 0V.

# CHAPTER 5: Arduino IDE Setup & Libraries

## 5.1  Install Arduino IDE

- Download Arduino IDE 2.x from: https://www.arduino.cc/en/software

- Install it (click Next, Next, Install like any normal software)

- Open Arduino IDE

## 5.2  Add ESP32 Board Support

- In Arduino IDE, go to File → Preferences

- In 'Additional boards manager URLs', paste this URL:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Click OK. Then go to Tools → Board → Boards Manager

- Search for 'esp32' → Install 'esp32 by Espressif Systems' (version 2.x or 3.x)

- Wait for installation (it downloads ~200MB)

- After install, go to Tools → Board → ESP32 Arduino → ESP32 Dev Module

## 5.3  Install Required Libraries

Go to Tools → Manage Libraries and install each of these:

| Library Name | Search Term | Purpose |
| --- | --- | --- |
| DHT sensor library | DHT sensor library | For DHT-22 temperature & humidity |
| Adafruit BMP280 | BMP280 | For barometric pressure sensor |
| MPU6050 by Electronic Cats | MPU6050 | For accelerometer/fall detection |
| Adafruit SSD1306 | SSD1306 | For OLED display |
| Adafruit GFX Library | Adafruit GFX | Required by SSD1306 |
| TinyGPS++ | TinyGPSPlus | For parsing GPS NMEA data |
| BlynkSimpleEsp32 | Blynk | For IoT dashboard |

## 5.4  Upload Settings

- Connect ESP32 to laptop via USB cable

- In Arduino IDE: Tools → Board → ESP32 Dev Module

- Tools → Port → Select the COM port (Windows: COM3, COM4 etc. Mac: /dev/cu.usbserial-*)

- Tools → Upload Speed → 115200

- Tools → Flash Size → 4MB

- Press the Upload button (right arrow icon)

- If upload fails: Hold the BOOT button on ESP32 while uploading, then release

💡 TIP: If you see 'Connecting...' for more than 10 seconds, hold the BOOT (or IO0) button on the ESP32 board and try again. This puts it in download mode.

# CHAPTER 6: The Complete ESP32 Code

This is the full code for the smart helmet. Copy it exactly into Arduino IDE. Read the comments (lines starting with //) to understand what each part does.

📝 NOTE: IMPORTANT: Replace 'YOUR_WIFI_NAME', 'YOUR_WIFI_PASSWORD', and 'YOUR_BLYNK_TOKEN' with your actual values before uploading.

### Part 1: Libraries and Definitions

// ============================================================

// SMART SAFETY HELMET FOR COAL MINERS

// Hardware: ESP32 + Multiple Sensors

// Author: [Your Name] | Confluence Project

// ============================================================

// --- LIBRARIES (must be installed via Library Manager) ---

#include <WiFi.h>              // ESP32 built-in WiFi

#include <BlynkSimpleEsp32.h>  // IoT Dashboard

#include <DHT.h>               // Temp & Humidity

#include <Adafruit_BMP280.h>   // Pressure Sensor

#include <MPU6050.h>           // Fall Detection

#include <Adafruit_SSD1306.h>  // OLED Display

#include <Adafruit_GFX.h>      // OLED Graphics

#include <TinyGPS++.h>         // GPS Parsing

#include <Wire.h>              // I2C Communication

// --- WiFi & Blynk Credentials ---

char auth[] = "YOUR_BLYNK_TOKEN";  // Get from Blynk app

char ssid[] = "YOUR_WIFI_NAME";

char pass[] = "YOUR_WIFI_PASSWORD";

// --- PIN DEFINITIONS ---

#define DHT_PIN      4    // DHT-22 data pin

#define DHT_TYPE     DHT22

#define MQ2_PIN      34   // Gas sensor analog (LPG/Smoke)

#define MQ7_PIN      32   // CO sensor analog

#define MQ135_PIN    33   // Air quality analog

#define O2_PIN       36   // Oxygen sensor analog

#define SW420_PIN    18   // Vibration sensor digital

#define BUZZER_PIN   26   // Active buzzer

#define SOS_BTN_PIN  5    // SOS push button

#define MPU_INT_PIN  19   // MPU6050 interrupt

// --- THRESHOLDS (adjust based on real calibration) ---

#define CO_THRESHOLD        300   // ADC value for dangerous CO

#define GAS_THRESHOLD       400   // ADC value for dangerous gas

#define AIR_QUALITY_THRESH  400   // ADC value for bad air

#define O2_LOW_THRESHOLD    850   // ADC value = ~19.5% O2

#define TEMP_MAX            40.0  // Celsius

#define PRESSURE_MIN        85000 // Pascals (below = too deep)

#define FALL_THRESHOLD      2.5   // G-force for fall detection

// --- OLED SETUP ---

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET   -1  // Use -1 if no reset pin

// --- OBJECT CREATION ---

DHT dht(DHT_PIN, DHT_TYPE);

Adafruit_BMP280 bmp;

MPU6050 mpu;

TinyGPSPlus gps;

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

HardwareSerial gpsSerial(2);  // UART2 for GPS

### Part 2: Variables

// --- GLOBAL VARIABLES ---

float temperature = 0;

float humidity = 0;

float pressure = 0;

float altitude = 0;

int   mq2Value = 0;

int   mq7Value = 0;

int   mq135Value = 0;

int   o2Value = 0;

float o2Percent = 0;

bool  sosTriggered = false;

bool  fallDetected = false;

bool  vibrationDetected = false;

double gpsLat = 0;

double gpsLng = 0;

// Blynk Virtual Pins (we assign each sensor a virtual pin)

// V0 = Temperature    V1 = Humidity      V2 = CO (MQ7)

// V3 = Gas (MQ2)      V4 = Air (MQ135)   V5 = Oxygen %

// V6 = Pressure       V7 = Altitude       V8 = SOS Alert

// V9 = Fall Alert     V10 = GPS Lat       V11 = GPS Lng

### Part 3: Setup Function

void setup() {

Serial.begin(115200);

Serial.println("Smart Helmet Booting...");

// --- PIN MODES ---

pinMode(BUZZER_PIN, OUTPUT);

pinMode(SOS_BTN_PIN, INPUT_PULLUP);  // Button pulls to GND when pressed

pinMode(SW420_PIN, INPUT);

digitalWrite(BUZZER_PIN, LOW);       // Buzzer OFF at start

// --- OLED INIT ---

Wire.begin(21, 22);  // SDA=21, SCL=22

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("ERROR: OLED not found!");

}

display.clearDisplay();

display.setTextColor(WHITE);

display.setTextSize(1);

display.setCursor(10, 20);

display.println("Smart Helmet");

display.setCursor(10, 35);

display.println("Initializing...");

display.display();

delay(2000);

// --- DHT INIT ---

dht.begin();

// --- BMP280 INIT ---

if (!bmp.begin(0x76)) {

Serial.println("ERROR: BMP280 not found! Check wiring.");

}

// --- MPU6050 INIT ---

mpu.initialize();

if (!mpu.testConnection()) {

Serial.println("ERROR: MPU6050 not found!");

}

// --- GPS INIT ---

gpsSerial.begin(9600, SERIAL_8N1, 16, 17);  // RX=16, TX=17

Serial.println("GPS initialized. Waiting for fix...");

// --- WIFI + BLYNK ---

Serial.print("Connecting to WiFi: ");

Serial.println(ssid);

Blynk.begin(auth, ssid, pass);

Serial.println("WiFi Connected!");

}

### Part 4: Main Loop & Sensor Reading

void loop() {

Blynk.run();              // Keep Blynk connection alive

readAllSensors();         // Read every sensor

checkThresholds();        // Check for danger

updateOLED();             // Update helmet display

sendToBlynk();            // Send data to cloud

readGPS();                // Parse GPS data

delay(2000);              // Wait 2 seconds between readings

}

// ============================================================

// READ ALL SENSORS

// ============================================================

void readAllSensors() {

// --- Temperature & Humidity ---

temperature = dht.readTemperature();

humidity    = dht.readHumidity();

if (isnan(temperature) || isnan(humidity)) {

Serial.println("DHT22 read failed!");

temperature = -999; humidity = -999;

}

// --- Pressure & Altitude ---

pressure = bmp.readPressure();    // In Pascals

altitude = bmp.readAltitude(1013.25); // Sea level pressure

// --- Gas Sensors (0-4095 ADC range on ESP32) ---

mq2Value   = analogRead(MQ2_PIN);

mq7Value   = analogRead(MQ7_PIN);

mq135Value = analogRead(MQ135_PIN);

o2Value    = analogRead(O2_PIN);

// Convert O2 ADC to percentage (calibrate with fresh air = 20.9%)

// This is a linear approximation — proper calibration needed

o2Percent = map(o2Value, 0, 4095, 0, 250) / 10.0;

// --- Vibration ---

vibrationDetected = digitalRead(SW420_PIN);

// --- SOS Button (LOW = pressed because of INPUT_PULLUP) ---

if (digitalRead(SOS_BTN_PIN) == LOW) {

sosTriggered = true;

triggerAlarm("SOS BUTTON PRESSED!");

}

// --- Fall Detection via MPU6050 ---

int16_t ax, ay, az;

mpu.getAcceleration(&ax, &ay, &az);

float gForce = sqrt(pow(ax/16384.0,2) + pow(ay/16384.0,2) + pow(az/16384.0,2));

if (gForce > FALL_THRESHOLD || gForce < 0.3) {

fallDetected = true;

triggerAlarm("FALL DETECTED!");

}

// --- Print to Serial for debugging ---

Serial.print("Temp: "); Serial.print(temperature); Serial.print("C  ");

Serial.print("Humid: "); Serial.print(humidity); Serial.print("%  ");

Serial.print("CO: "); Serial.print(mq7Value); Serial.print("  ");

Serial.print("O2: "); Serial.print(o2Percent); Serial.println("%");

}

### Part 5: Threshold Check & Alarm

// ============================================================

// CHECK ALL THRESHOLDS AND TRIGGER ALARMS

// ============================================================

void checkThresholds() {

bool danger = false;

String dangerMsg = "";

if (mq7Value > CO_THRESHOLD) {

danger = true;

dangerMsg += "HIGH CO! ";

Blynk.logEvent("co_alert", "Dangerous CO level detected!");

}

if (mq2Value > GAS_THRESHOLD) {

danger = true;

dangerMsg += "FLAMMABLE GAS! ";

Blynk.logEvent("gas_alert", "Flammable gas detected!");

}

if (mq135Value > AIR_QUALITY_THRESH) {

danger = true;

dangerMsg += "BAD AIR! ";

}

if (o2Value < O2_LOW_THRESHOLD) {

danger = true;

dangerMsg += "LOW O2! ";

Blynk.logEvent("o2_alert", "Low Oxygen level!");

}

if (temperature > TEMP_MAX) {

danger = true;

dangerMsg += "HIGH TEMP! ";

}

if (vibrationDetected) {

danger = true;

dangerMsg += "VIBRATION! ";

Blynk.logEvent("vibration_alert", "Mine vibration detected!");

}

if (sosTriggered) {

danger = true;

dangerMsg += "SOS! ";

Blynk.logEvent("sos", "SOS activated by miner!");

sosTriggered = false;  // Reset after sending

}

if (fallDetected) {

danger = true;

dangerMsg += "FALL! ";

Blynk.logEvent("fall_alert", "Miner fall detected!");

fallDetected = false;

}

if (danger) triggerAlarm(dangerMsg);

else digitalWrite(BUZZER_PIN, LOW);  // No danger, buzzer off

}

void triggerAlarm(String msg) {

Serial.println("ALARM: " + msg);

digitalWrite(BUZZER_PIN, HIGH);  // Sound alarm

delay(500);

digitalWrite(BUZZER_PIN, LOW);

delay(200);

digitalWrite(BUZZER_PIN, HIGH);

delay(500);

digitalWrite(BUZZER_PIN, LOW);

}

### Part 6: OLED Display & Blynk

// ============================================================

// UPDATE OLED DISPLAY ON HELMET

// ============================================================

void updateOLED() {

display.clearDisplay();

display.setTextSize(1);

display.setCursor(0, 0);

display.print("T:"); display.print(temperature,1);

display.print("C H:"); display.print(humidity,1); display.println("%");

display.print("CO:"); display.print(mq7Value);

display.print(" GAS:"); display.println(mq2Value);

display.print("O2:"); display.print(o2Percent,1); display.println("%");

display.print("P:"); display.print(pressure/100,0); display.println("hPa");

if (sosTriggered) {

display.setTextSize(2);

display.println("  SOS!");

}

display.display();

}

// ============================================================

// SEND DATA TO BLYNK DASHBOARD

// ============================================================

void sendToBlynk() {

Blynk.virtualWrite(V0, temperature);

Blynk.virtualWrite(V1, humidity);

Blynk.virtualWrite(V2, mq7Value);    // CO

Blynk.virtualWrite(V3, mq2Value);    // Gas

Blynk.virtualWrite(V4, mq135Value);  // Air Quality

Blynk.virtualWrite(V5, o2Percent);   // O2 %

Blynk.virtualWrite(V6, pressure/100);// hPa

Blynk.virtualWrite(V7, altitude);

if (gpsLat != 0) {

Blynk.virtualWrite(V10, gpsLat);

Blynk.virtualWrite(V11, gpsLng);

}

}

// ============================================================

// READ GPS DATA

// ============================================================

void readGPS() {

while (gpsSerial.available() > 0) {

gps.encode(gpsSerial.read());

}

if (gps.location.isValid()) {

gpsLat = gps.location.lat();

gpsLng = gps.location.lng();

Serial.print("GPS: "); Serial.print(gpsLat, 6);

Serial.print(", "); Serial.println(gpsLng, 6);

}

}

💡 TIP: Upload this code, open Serial Monitor at 115200 baud, and watch sensor readings scroll. If a sensor shows -999 or 0 constantly, check its wiring and power supply.

# CHAPTER 7: IoT Dashboard Setup (Blynk)

Blynk is a free app that turns your phone into a live dashboard. Supervisors can monitor all miners from one screen, and the app sends notifications when danger is detected.

## 7.1  Create Blynk Account

- Download 'Blynk IoT' app on Android or iPhone

- Go to blynk.cloud and create a free account

- Click 'New Template' → Name it 'Smart Helmet' → Choose 'ESP32' hardware → 'WiFi' connection

- Click 'Create'

## 7.2  Create Datastreams

Datastreams connect the ESP32's virtual pins to dashboard widgets:

- In Blynk, go to your template → 'Datastreams' tab

- Create New Datastream for each sensor:

| Pin | Name | Data Type | Range | Units |
| --- | --- | --- | --- | --- |
| V0 | Temperature | Double | -40 to 80 | Celsius (°C) |
| V1 | Humidity | Double | 0 to 100 | % RH |
| V2 | CO Level | Integer | 0 to 4095 | ADC units |
| V3 | Gas (LPG) | Integer | 0 to 4095 | ADC units |
| V4 | Air Quality | Integer | 0 to 4095 | ADC units |
| V5 | Oxygen % | Double | 0 to 25 | % O2 |
| V6 | Pressure | Double | 300 to 1100 | hPa |
| V7 | Altitude | Double | -500 to 9000 | meters |
| V10 | GPS Latitude | Double | -90 to 90 | degrees |
| V11 | GPS Longitude | Double | -180 to 180 | degrees |

## 7.3  Build the Dashboard

- Go to 'Web Dashboard' in Blynk → Add Widgets:

- Gauge widget → V0 (Temperature) → Set red zone above 40°C

- Gauge widget → V5 (Oxygen %) → Set red zone below 19.5%

- Chart widget → V2 (CO Level) → Shows history graph

- Chart widget → V3 (Gas Level) → Shows history graph

- Value Display → V6 (Pressure)

- Value Display → V7 (Altitude)

- LED widget → Flash red when any alert event occurs

- Map widget → V10 + V11 for GPS location (shows miner on map!)

## 7.4  Set Up Notifications

- In Blynk Template → go to 'Events' tab

- Create events: 'co_alert', 'gas_alert', 'o2_alert', 'fall_alert', 'sos', 'vibration_alert'

- For each event, enable 'Push Notifications' → This sends phone alerts!

- Enable 'Email' notifications for critical events like SOS and fall_alert

- Set 'Event Rate Limit' to 1 minute to avoid spam

💡 TIP: Get your Blynk Auth Token: In Blynk web → your Device → Device Info → Auth Token. Copy this and paste it in the code as auth[].

# CHAPTER 8: Testing & Troubleshooting

## 8.1  Testing Each Sensor

### Test 1: DHT-22 Temperature & Humidity

- Upload the code, open Serial Monitor

- You should see temperature (~25°C) and humidity (~50%)

- Hold the sensor in your fist for 30 seconds → temperature should rise

- Blow on it → humidity should increase slightly

### Test 2: Gas Sensors (MQ-2, MQ-7, MQ-135)

- Let sensors warm up for 5 minutes after powering on

- Hold a lighter (WITHOUT flame, just gas) near MQ-2 → value should spike

- Light an incense stick and hold near MQ-135 → air quality value rises

⚠️ WARNING: DO NOT use actual CO gas for testing. For classroom testing, breathe gently on the sensor — CO2 in breath increases MQ-135 reading.

### Test 3: Fall Detection (MPU-6050)

- Hold the breadboard level → g-force should be ~1.0 G

- Quickly tilt it 90 degrees → acceleration values change

- Simulate a 'drop' by quickly moving it downward (do not actually drop it!)

- The fall alarm should trigger if g-force exceeds threshold

### Test 4: SOS Button

- Press the push button → Serial Monitor shows 'SOS BUTTON PRESSED!'

- Buzzer should sound twice

- Blynk app should show SOS notification

### Test 5: GPS

- Take the setup near a window or outdoors

- Wait 1-2 minutes for satellite fix (blue LED on NEO-6M blinks at 1Hz)

- Serial Monitor shows latitude and longitude values

- Open Google Maps, type the coordinates → should show your location

## 8.2  Common Problems & Solutions

| Problem | Likely Cause | Solution |
| --- | --- | --- |
| Upload fails - 'Connecting...' | ESP32 not in download mode | Hold BOOT button during upload |
| DHT22 shows nan or -999 | Pull-up resistor missing or wrong pin | Add 10kΩ between DATA and 3.3V |
| Gas sensor always 4095 | Voltage divider missing | Add 10kΩ+10kΩ divider on AOUT |
| BMP280/MPU not found | Wrong I2C address | Try 0x77 for BMP, 0x69 for MPU |
| GPS never gets fix | No antenna or indoors | Move near window, wait 2 minutes |
| OLED blank | Wrong I2C address or no power | Check 3.3V supply, try address 0x3D |
| Blynk disconnects often | Weak WiFi signal | Move closer to router, check SSID/pass |
| Buzzer always ON | Threshold too low or sensor not warmed up | Increase threshold values in code |
| ESP32 restarting | Insufficient power | Use powered USB hub or 5V 2A adapter |

## 8.3  Sensor Calibration Tips

- MQ Sensors: After first power-on, let them run for 24 hours before trusting values. This 'burn-in' stabilizes the heater.

- DHT-22: Wait 2 seconds between readings — the sensor needs time to settle.

- MPU-6050: Run calibration code first to find offset values for your specific chip. Every chip has slightly different zero-point errors.

- O2 Sensor (KE-25): Calibrate in fresh air. Fresh air = 20.9% O2. Record the raw ADC value in fresh air and use it as your 100% reference.

- BMP280 Altitude: The altitude reading depends on knowing the sea-level pressure at your location (changes daily with weather). Use a weather app to find local sea-level pressure and update the 1013.25 value in bmp.readAltitude().

# CHAPTER 9: Final Project Checklist

Use this checklist before your Confluence presentation to make sure everything is working:

## Hardware Checklist

- All sensors connected and powered correctly

- No loose wires — press each wire firmly into the breadboard

- Voltage dividers in place for all MQ sensors

- GPS antenna attached and positioned for maximum sky exposure

- OLED display showing sensor readings

- Buzzer working — test by lowering threshold temporarily

- SOS button tested and working

- Battery connected and ESP32 powered independently of USB

## Software Checklist

- WiFi credentials correct

- Blynk token correct and matching

- All libraries installed

- Code uploaded successfully without errors

- Serial Monitor shows sensible values for all sensors

- No sensor shows -999 or 0 constantly

## Dashboard Checklist

- Blynk app open on phone shows live data

- All gauges and graphs updating every 2 seconds

- Test alert: trigger SOS button → phone notification received

- GPS location showing on map widget

- History graph showing data from last hour

## Presentation Tips

- Demo LIVE: Show the sensors responding in real time. Hold a lighter near MQ-2 and show the gas level spike on the dashboard.

- Explain WHY each sensor: Judges love when you explain the science behind each sensor choice.

- Show the problem first: Start with a 30-second explanation of how many miners die each year from gas incidents.

- Mention ESP32 advantage: Compare to the Arduino-based reference project and show why ESP32 is better.

- Have backup: Take a short video of the working system in case of technical issues during presentation.

Good luck at Confluence!

This project can genuinely save lives. Build it with care.