CONFLUENCE  2026

Technical Project Report

IoT-Based Smart Safety Helmet

for Coal Miners

Using ESP32 Microcontroller  •  IoT Cloud  •  Multi-Sensor Integration

| Domain IoT / Embedded Systems | Category Safety & Industrial IoT |
| --- | --- |
| Hardware ESP32 + 9 Sensors | Dashboard Blynk IoT / ThingSpeak |

Anand Ambastha  • Department of  Electronics and Communication •   Bharati Vidyapeeths’ COE •   April 2026

# Table of Contents

# Abstract

Coal mining remains one of the most hazardous occupations worldwide, with workers constantly exposed to toxic gases, oxygen deficiency, extreme temperatures, structural collapses, and fall-related injuries. This paper presents the design, implementation, and evaluation of an Internet of Things (IoT) based Smart Safety Helmet for coal miners, built around the ESP32 dual-core microcontroller. The system integrates nine heterogeneous sensors — covering gas detection (MQ-2, MQ-7, MQ-135), oxygen monitoring (KE-25), environmental parameters (DHT-22, BMP-280), motion and fall detection (MPU-6050, SW-420), and GPS tracking (NEO-6M) — along with an SOS emergency trigger. Real-time data is transmitted over WiFi to the Blynk IoT cloud dashboard, enabling remote monitoring by supervisors with push-notification alerts for threshold violations. A 0.96" OLED display provides on-helmet feedback to the miner. Experimental results demonstrate reliable multi-parameter sensing with end-to-end latency below 3 seconds. The proposed system offers a cost-effective, scalable, and deployable solution for industrial worker safety.

Keywords: IoT, ESP32, Smart Helmet, Coal Mine Safety, Gas Detection, Fall Detection, GPS Tracking, Blynk, Real-Time Monitoring

# 1  Introduction

### 1.1  Background and Motivation

Coal mining contributes significantly to global energy production, yet it continues to claim thousands of lives annually. According to the International Labour Organization, the mining sector accounts for about 8% of workplace fatalities despite employing less than 1% of the global workforce. Underground coal mines pose multiple simultaneous hazards: methane and carbon monoxide gases accumulate rapidly, oxygen levels can plummet without warning, structural collapses occur without advance notice, and high temperatures impair worker judgment and physical capacity.

Conventional safety measures — periodic manual gas readings, fixed-position sensors, verbal communication — are reactive and geographically constrained. A miner who collapses from CO poisoning deep in a tunnel may not be found for hours. The integration of sensor technology, microcontrollers, and wireless communication into personal protective equipment (PPE) such as helmets offers a proactive, continuously active safety layer that travels with the miner.

### 1.2  Problem Statement

The core problem addressed by this project is the lack of real-time, continuous, and location-aware safety monitoring for individual coal miners. Existing solutions either require expensive infrastructure (wired sensor networks), are geographically fixed, or provide only single-parameter monitoring. A comprehensive, wearable, internet-connected safety device that simultaneously monitors gas levels, oxygen, temperature, pressure, motion, and location — and alerts both the miner and remote supervisors instantly — does not exist as an affordable, off-the-shelf product accessible to small and mid-scale mining operations.

### 1.3  Objectives

- Design a wearable multi-sensor node embedded within a standard mining helmet

- Monitor six safety-critical parameters simultaneously: toxic gas levels, oxygen concentration, temperature, humidity, barometric pressure, and inertial motion

- Detect falls and sudden vibrations (indicative of mine collapse) in real time

- Provide GPS coordinates of the miner to a remote dashboard

- Transmit all data to an IoT cloud platform with configurable alert thresholds

- Display live readings on an OLED screen worn by the miner

- Achieve end-to-end alert latency below 5 seconds

### 1.4  Scope and Limitations

This project covers hardware design, firmware development, cloud integration, and bench-level testing. The current prototype operates over standard 2.4 GHz WiFi, which limits functionality to mine entrances or WiFi-enabled tunnels. Deep-underground operation would require LoRa or mesh radio extensions, which are identified as future work. Sensor calibration is performed under laboratory conditions; mine-environment calibration will require extended field deployment.

# 2  Literature Review

### 2.1  Existing Work

Early smart helmet research (Yarlagadda et al., 2017) focused on Arduino Uno platforms with DHT11 and MQ-series gas sensors, transmitting data via ESP8266 WiFi modules. While demonstrating feasibility, these designs suffered from limited processing power, a restricted number of analog inputs, and the complexity of integrating two separate microcontroller boards.

Subsequent work by Raj et al. (2019) introduced GPS tracking and SMS-based alerts via GSM modules (SIM800L), significantly improving location awareness but adding cost and power consumption. Ramesh and Kumar (2020) explored LoRa-based long-range communication suitable for tunnels without WiFi infrastructure. Chen et al. (2021) incorporated machine learning on edge devices to distinguish fall events from normal motion using MPU-6050 data.

### 2.2  Research Gap

A review of existing literature reveals that no single published prototype integrates all of the following simultaneously: multi-gas detection, electrochemical oxygen monitoring, environmental sensors, IMU-based fall detection, vibration sensing, GPS tracking, IoT cloud dashboard, and on-device display — within a single, compact, low-cost ESP32-based platform with real-time push notifications. This project addresses that gap.

### 2.3  Why ESP32 Over Prior Architectures

| Criterion | Justification for ESP32 |
| --- | --- |
| Processing Power | Dual-core Xtensa LX6 at 240 MHz handles concurrent sensor polling, GPS parsing, and WiFi transmission without bottlenecks. Single-core AVR (Arduino) struggles under this load. |
| Connectivity | Built-in 802.11b/g/n WiFi and Bluetooth 4.2 / BLE eliminates the need for an external ESP8266 module, reducing BOM cost and wiring complexity. |
| Analog Inputs | 18 ADC channels vs. Arduino Uno's 6, enabling connection of all MQ sensors without multiplexers. |
| I2C/UART Buses | Multiple hardware I2C and UART peripherals allow simultaneous communication with BMP280, MPU6050, OLED (I2C) and NEO-6M GPS (UART2) without bus conflicts. |
| FreeRTOS Support | Native FreeRTOS enables multi-tasking: one task handles sensor reads, another handles WiFi/Blynk, ensuring consistent sampling intervals even during cloud transmission. |
| Power Management | Deep-sleep modes can reduce current to ~10 µA, extending battery life between shifts — not available on Arduino. |

# 3  System Architecture

### 3.1  Block Diagram Overview

The system is structured into three functional layers: the Sensing Layer (all sensors interfacing to ESP32), the Processing Layer (ESP32 firmware performing data acquisition, threshold checking, and display), and the Communication Layer (WiFi → Blynk Cloud → Supervisor Dashboard).

| Layer | Components | Function |
| --- | --- | --- |
| Sensing | MQ-2, MQ-7, MQ-135, KE-25, DHT-22, BMP-280, MPU-6050, SW-420, NEO-6M, Push Button | Measure physical/chemical properties of the environment and miner's state |
| Processing | ESP32-WROOM-32 (240 MHz dual-core, FreeRTOS) | Read sensors, compare against thresholds, drive buzzer and OLED, prepare data packets |
| Output (local) | 0.96" SSD1306 OLED, Active Buzzer | Display real-time readings to miner; sound alarm on threshold breach |
| Communication | ESP32 WiFi → Blynk Cloud → Supervisor App | Transmit live telemetry; receive acknowledgement; send event alerts (push/email) |
| Power | 3.7 V Li-Po 2000 mAh + TP4056 BMS | Portable power with over-charge and over-discharge protection |

### 3.2  Communication Protocol Selection

| Protocol | Role in This System |
| --- | --- |
| I2C (GPIO 21/22) | BMP-280, MPU-6050, SSD1306 OLED — multi-device shared bus |
| UART2 (GPIO 16/17) | NEO-6M GPS module — 9600 baud NMEA stream |
| ADC (GPIOs 32–36) | MQ-2, MQ-7, MQ-135, KE-25 — 12-bit analog readings (0–4095) |
| Digital I/O | DHT-22 single-wire, SW-420 digital out, Buzzer, SOS button |
| 802.11 WiFi | Blynk cloud uplink at 2-second intervals |

# 4  Hardware Design

### 4.1  Sensor Specifications and Selection Rationale

| Sensor | Parameter | Interface | Selection Rationale |
| --- | --- | --- | --- |
| MQ-2 | LPG / Smoke / Methane | Analog (ADC) | Detects explosive gases. Concentration range 300–10,000 ppm. Low cost; widely calibrated for coal mine hazards. |
| MQ-7 | Carbon Monoxide (CO) | Analog (ADC) | CO is the leading cause of mine fatalities. MQ-7 uses a duty-cycle heating scheme (5V / 1.5V) for improved CO selectivity over other MQ sensors. |
| MQ-135 | Air Quality / NH3 / CO2 | Analog (ADC) | Monitors overall air quality. Detects ammonia released from blasting agents and general air quality degradation. |
| KE-25 / SEN0340 | Oxygen (O2 %) | Analog (ADC) | Electrochemical cell — far more accurate than semiconductor O2 sensors. Critical for detecting O2 deficiency (threshold: 19.5%). Includes signal amplification on breakout board. |
| DHT-22 | Temperature + Humidity | Single-wire digital | Range: −40 to +80 °C, 0–100% RH. Superior accuracy vs. DHT-11 (±0.5 °C vs. ±2 °C). Essential for heat-stroke prevention. |
| BMP-280 | Pressure + Altitude | I2C | Detects rapid pressure changes indicative of mine collapse. Altitude helps map depth of miner. Ultra-low power (2.7 µA in normal mode). |
| MPU-6050 | Acceleration + Gyroscope | I2C | 6-DOF IMU enables fall detection via sudden acceleration change algorithm and tilt monitoring for unconsciousness detection. |
| SW-420 | Vibration | Digital | Detects ground vibrations from blasting or structural shifts. Adjustable sensitivity. Provides early warning before collapse. |
| NEO-6M | GPS Location | UART | Sub-metre accuracy. 50-channel receiver. Provides lat/lng to cloud dashboard for rescue mapping. Works at mine entrance and shaft head. |

### 4.2  Pin Assignment Summary

| Component | ESP32 Pin | Signal Type | Notes |
| --- | --- | --- | --- |
| DHT-22 | GPIO 4 | Digital 1-wire | 10 kΩ pull-up to 3.3 V required |
| MQ-2 AOUT | GPIO 34 | ADC (12-bit) | Voltage divider: 10 kΩ + 10 kΩ — max 2.5 V on pin |
| MQ-7 AOUT | GPIO 32 | ADC (12-bit) | Same voltage divider rule |
| MQ-135 AOUT | GPIO 33 | ADC (12-bit) | Same voltage divider rule |
| KE-25 (O2) | GPIO 36 | ADC (12-bit) | Input-only pin; no pull-up |
| BMP-280 SDA/SCL | GPIO 21 / 22 | I2C | Shared bus with MPU & OLED; 4.7 kΩ pull-ups |
| MPU-6050 SDA/SCL | GPIO 21 / 22 | I2C | Address 0x68; INT → GPIO 19 |
| OLED SSD1306 | GPIO 21 / 22 | I2C | Address 0x3C |
| NEO-6M TX→RX | GPIO 16 | UART2 RX | 9600 baud; GPS TX → ESP32 RX |
| NEO-6M RX←TX | GPIO 17 | UART2 TX |  |
| SW-420 | GPIO 18 | Digital INPUT | Active HIGH on vibration |
| SOS Button | GPIO 5 | Digital INPUT_PULLUP | Active LOW; external pull-up not needed |
| Buzzer (+) | GPIO 26 | Digital OUTPUT | Active buzzer; no resistor needed |

### 4.3  Power Supply Design

The system is powered by a 3.7 V, 2000 mAh lithium polymer (Li-Po) battery connected through a TP4056 module with integrated battery management (BMS). The TP4056 provides constant-current/constant-voltage (CC/CV) charging at 1 A when connected to a 5 V USB source and protects against overcharge (> 4.2 V), over-discharge (< 3.0 V), and short circuit.

The ESP32 DevKit board includes an onboard LDO regulator that produces a stable 3.3 V rail for I2C sensors, GPS, and digital peripherals. MQ-series gas sensors require 5 V for their internal heating elements; these are powered from the USB 5 V (Vin) rail when tethered to USB during development, or from a separate 5 V boost converter from the Li-Po during field deployment.

🔋  Estimated Battery Life

At full operation (all sensors + WiFi active + OLED on), measured current draw is approximately 280 mA. A 2000 mAh battery provides ~7.1 hours of continuous operation — sufficient for an 8-hour mining shift with margin. Deep-sleep between readings can extend this to 12+ hours.

# 5  Firmware Design

### 5.1  Software Architecture

The firmware is developed in C++ using the Arduino framework for ESP32 (Espressif Arduino Core v3.x). The main loop executes a fixed 2-second cycle: read all sensors → evaluate thresholds → update OLED → transmit to Blynk → parse GPS. The Blynk.run() call is included in every loop iteration to maintain the cloud connection.

### 5.2  Threshold Logic

Each sensor parameter is compared against a predefined danger threshold derived from industry standards (OSHA, MSHA guidelines):

| Parameter | Safe Limit | Danger Threshold | Standard Reference |
| --- | --- | --- | --- |
| Carbon Monoxide | < 25 ppm TWA | ADC > 300 (≈50 ppm) | OSHA PEL: 50 ppm |
| Oxygen Level | > 19.5% | < 19.5% O2 | OSHA 1910.146 |
| Methane / LPG | < 1% LEL | ADC > 400 | MSHA 30 CFR |
| Air Quality | AQI < 100 | ADC > 400 | General industrial |
| Temperature | < 38 °C WBGT | Temp > 40 °C | ISO 7933 |
| Barometric Pressure | Sea-level ± 15 hPa | < 850 hPa absolute | Site-specific |
| Fall Detection | G-force ~1.0 G | G-force > 2.5 G | Published IMU fall algo. |

### 5.3  Fall Detection Algorithm

Fall detection uses the resultant acceleration magnitude calculated from the three accelerometer axes:

G = √(ax² + ay² + az²)  /  16384

A fall event is declared when G > 2.5 (impact phase) or G < 0.3 (free-fall phase). After a fall is detected, the system waits 3 seconds and re-checks: if G remains < 0.6 (miner lying still), a secondary 'motionless' alert is sent to the cloud, indicating possible unconsciousness.

### 5.4  GPS Data Pipeline

The NEO-6M module streams NMEA sentences at 9600 baud over UART2. The TinyGPS++ library parses these in real time inside the main loop. When a valid fix is obtained (location.isValid() == true), the latitude and longitude are written to Blynk virtual pins V10 and V11. The Blynk map widget plots the miner's position on a satellite map visible to supervisors.

### 5.5  Libraries Used

| Library | Version | Purpose |
| --- | --- | --- |
| ESP32 Arduino Core (Espressif) | 3.x | Board support, WiFi, ADC, UART |
| BlynkSimpleEsp32 | 1.3.x | IoT cloud connectivity and virtual pins |
| DHT sensor library (Adafruit) | 1.4.x | DHT-22 driver |
| Adafruit BMP280 Library | 2.6.x | Pressure and temperature from BMP280 |
| MPU6050 (Electronic Cats) | 1.2.x | 6-axis IMU accelerometer/gyroscope |
| Adafruit SSD1306 | 2.5.x | OLED display driver |
| Adafruit GFX Library | 1.11.x | Graphics primitives for OLED |
| TinyGPS++ | 1.0.x | NMEA GPS sentence parsing |

# 6  IoT Dashboard Design

### 6.1  Platform Selection: Blynk IoT

Blynk IoT (blynk.cloud) was selected as the primary cloud platform due to its free tier, native ESP32 support, mobile application for real-time monitoring, configurable event-based push notifications, and map widget for GPS visualization. The free Blynk plan supports up to 10 datastreams and 5 event types — sufficient for this project.

### 6.2  Dashboard Widgets

| Virtual Pin | Widget Type | Data Source | Configuration |
| --- | --- | --- | --- |
| V0 | Gauge | Temperature | Range −10 to 60°C; red zone > 40°C |
| V1 | Gauge | Humidity | Range 0–100%; informational only |
| V2 | Chart (Line) | CO Level | 1-hour history; red threshold line at danger ADC value |
| V3 | Chart (Line) | Gas (MQ-2) | 1-hour history; threshold line |
| V4 | Value Display | Air Quality | Numeric; color: green < 200, amber < 400, red > 400 |
| V5 | Gauge | Oxygen % | Range 0–25%; red zone < 19.5% |
| V6 | Value Display | Pressure (hPa) | Numeric; alert if < 850 hPa |
| V7 | Value Display | Altitude (m) | Numeric; depth reference |
| V8 | LED | SOS Alert | Blinks red on SOS event |
| V9 | LED | Fall Alert | Blinks red on fall event |
| V10 + V11 | Map | GPS Lat / Lng | Live pin shows miner position |

### 6.3  Alert Event Configuration

Six events are configured in the Blynk Template with the following notification channels:

- co_alert — Push notification + Email (Priority: HIGH)

- gas_alert — Push notification + Email (Priority: HIGH)

- o2_alert — Push notification + Email (Priority: CRITICAL)

- fall_alert — Push notification + SMS (Priority: CRITICAL)

- sos — Push notification + Email + SMS (Priority: CRITICAL)

- vibration_alert — Push notification (Priority: MEDIUM)

Event rate limiting is set to 1 event per minute per type to prevent notification flooding during sustained alarm conditions.

# 7  Results and Testing

### 7.1  Sensor Performance

| Sensor | Test Method | Expected | Measured | Result |
| --- | --- | --- | --- | --- |
| DHT-22 | Thermometer / Hygrometer reference | 25.0°C / 55% | 24.8°C / 54.2% | PASS (±0.5°C) |
| BMP-280 | Known altitude location | 57 m (test location) | 61 m | PASS (±5 m) |
| MPU-6050 | Controlled drop test (30 cm) | G spike > 2.5 | G = 3.1 detected | PASS |
| SW-420 | Table knock vibration test | HIGH output | Triggered reliably | PASS |
| MQ-2 | Lighter gas (no flame) near sensor | ADC rise | ADC 110 → 2800 | PASS |
| MQ-7 | Breath (CO₂) proximity test | Mild ADC rise | ADC 95 → 340 | PASS |
| NEO-6M | Outdoor fix verification | Valid GPS coords | Fix in 45 sec | PASS |
| SOS Button | Manual press | Alert triggered | Alert in < 1 s | PASS |

### 7.2  System Latency

| Measurement | Value |
| --- | --- |
| Sensor read cycle (all sensors) | ≈ 380 ms |
| OLED update time | ≈ 45 ms |
| Blynk virtualWrite (11 pins) | ≈ 120 ms |
| End-to-end (sensor → phone notification) | 1.8 – 2.9 s |
| GPS first fix (outdoor, clear sky) | 38 – 62 s |
| SOS button → notification | < 1.2 s |

✅  Key Finding

All measured end-to-end alert latencies are well below the 5-second design target. The system successfully detected simulated gas exposure, fall events, SOS activation, and vibration in all 10 repeated trials during bench testing.

# 8  Cost Analysis

### 8.1  Bill of Materials

| # | Component | Model | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- | --- |
| 1 | ESP32 Microcontroller | WROOM-32 DevKit | 1 | ₹ 400 | ₹ 400 |
| 2 | Gas Sensor (LPG/Smoke) | MQ-2 | 1 | ₹ 100 | ₹ 100 |
| 3 | Carbon Monoxide Sensor | MQ-7 | 1 | ₹ 140 | ₹ 140 |
| 4 | Air Quality Sensor | MQ-135 | 1 | ₹ 100 | ₹ 100 |
| 5 | Oxygen Sensor + Breakout | DFR SEN0340 (KE-25) | 1 | ₹ 1200 | ₹ 1200 |
| 6 | Temperature + Humidity | DHT-22 | 1 | ₹ 180 | ₹ 180 |
| 7 | Barometric Pressure | BMP-280 | 1 | ₹ 150 | ₹ 150 |
| 8 | IMU (Accel + Gyro) | MPU-6050 | 1 | ₹ 100 | ₹ 100 |
| 9 | Vibration Sensor | SW-420 | 1 | ₹ 45 | ₹ 45 |
| 10 | GPS Module + Antenna | NEO-6M | 1 | ₹ 450 | ₹ 450 |
| 11 | OLED Display | SSD1306 0.96" I2C | 1 | ₹ 150 | ₹ 150 |
| 12 | Active Buzzer | 5 V Piezo | 1 | ₹ 30 | ₹ 30 |
| 13 | Push Button (SOS) | Momentary NO | 2 | ₹ 10 | ₹ 20 |
| 14 | Li-Po Battery | 3.7 V 2000 mAh | 1 | ₹ 350 | ₹ 350 |
| 15 | Battery Charger Module | TP4056 BMS | 1 | ₹ 35 | ₹ 35 |
| 16 | Miscellaneous (resistors, wires, PCB) | — | — | ₹ 200 | ₹ 200 |
| TOTAL PROTOTYPE COST |  |  |  |  | ₹ 3,650 |

The total prototype cost of approximately ₹3,650 (USD ~44) compares favourably to commercial industrial gas detectors (₹15,000–₹80,000) and GPS tracking vests (₹10,000–₹30,000), while offering more integrated functionality.

# 9  Conclusion and Future Work

### 9.1  Conclusion

This project successfully demonstrates an IoT-based smart safety helmet for coal miners using the ESP32 microcontroller. The system integrates nine sensors covering all major underground hazards, communicates over WiFi to a cloud dashboard, and provides both on-device feedback and remote push notifications. Bench testing confirms reliable multi-parameter sensing with end-to-end alert latency below 3 seconds and a total prototype cost under ₹4,000 — making it significantly more affordable and feature-rich than commercially available alternatives.

The project validates the hypothesis that a single ESP32 board, with appropriate sensor selection and firmware design, can serve as a comprehensive wearable safety platform for industrial workers, replacing multi-board architectures used in prior work.

### 9.2  Future Work

- LoRa / Mesh Radio Integration: Replace WiFi with LoRa (SX1278) or a sub-GHz mesh network to extend underground coverage beyond WiFi range

- Machine Learning for Fall Detection: Train a neural network on MPU-6050 data to distinguish actual falls from tool-handling vibrations, reducing false positives

- PCB Design and Miniaturization: Migrate from breadboard to a custom PCB sized to fit inside a standard ABS mining helmet shell

- Multi-Miner Dashboard: Extend Blynk dashboard to track multiple miners simultaneously with colour-coded status indicators per worker

- Edge AI Gas Classification: Deploy a TensorFlow Lite model on ESP32 to identify specific gas compounds from MQ sensor array patterns, improving identification accuracy

- Solar Trickle Charging: Add a small flexible solar panel on the helmet exterior to extend battery life for long-shift deployments

- Offline Data Logging: Write sensor readings to onboard SD card for post-shift analysis even when WiFi is unavailable

# References

[1]  Yarlagadda, A. et al. (2017). IoT Based Smart Helmet for Mine Workers. International Journal of Engineering Research & Technology, 6(5).

[2]  Raj, S. et al. (2019). Smart Safety Helmet for Coal Mine Workers Using IoT. Proceedings of the IEEE ICCMC 2019.

[3]  Ramesh, K. & Kumar, A. (2020). LoRa-Based Underground Mine Safety Monitoring System. IEEE Sensors Journal, 20(14).

[4]  Chen, X. et al. (2021). Edge-Based Fall Detection for Mining Personnel Using IMU and Neural Networks. Sensors, 21(8), 2840.

[5]  International Labour Organization (2023). Safety and Health at Work: Global Estimates. ILO Report.

[6]  Espressif Systems (2024). ESP32 Technical Reference Manual v5.1. Espressif Systems.

[7]  OSHA (2023). Permissible Exposure Limits for Mining Atmospheres. U.S. Department of Labor.

[8]  Blynk Inc. (2024). Blynk IoT Platform Documentation. https://docs.blynk.io

[9]  TinyGPS++ Library. (2024). https://github.com/mikalhart/TinyGPSPlus

[10]  Adafruit Industries (2024). MPU-6050 and BMP-280 Libraries. https://github.com/adafruit