Bharati Vidyapeeth College of Engineering

CONFLUENCE 2026  —  Project Report

IoT Smart Safety Helmet for Coal Miners Using ESP32 & IoT Cloud Dashboard

| Group Members Sonvi Aayush Anand Ananya Suman | Institution Bharati Vidyapeeth College of Engineering Microcontroller: ESP32 Platform: Blynk IoT / ThingSpeak |
| --- | --- |

| 10 Sensors | ₹3,690 Total Cost | 2 sec Update Interval | ~6 hrs Battery Life |
| --- | --- | --- | --- |

# Chapter 1: Introduction & Project Overview

## 1.1 What Are We Building?

The IoT Smart Safety Helmet is a real-time environmental monitoring system for coal miners. Built around the ESP32 microcontroller, it uses 10 different sensors to continuously monitor dangerous underground conditions and instantly alerts both the miner and supervisors through cloud-connected dashboards, buzzer alarms, SMS, and email notifications.

Think of it as a traditional hard hat that has been given a brain — a helmet that reads the environment every 2 seconds and shouts for help the moment something dangerous happens.

## 1.2 Why Is This Important?

- Coal mines contain invisible, odorless gases like Carbon Monoxide (CO) and Methane that cannot be detected by human senses alone

- Oxygen levels can drop to fatal levels (below 6%) without any visible warning

- A slip or fall can trap an unconscious miner with no way to alert anyone

- Temperature and pressure extremes underground can cause heat stroke or equipment failure

- Rescuers often don't know the exact location of a trapped miner during emergencies

- Traditional helmets are passive — they protect only from physical impacts, not environmental hazards

## 1.3 Our Solution

SMART HELMET SYSTEM OVERVIEW

INPUT (10 Sensors) → PROCESSING (ESP32) → OUTPUT (Alerts + Dashboard) Sensors monitor: CO, Gas/Smoke, Oxygen, Air Quality, Temperature, Humidity, Pressure, Fall, Vibration, GPS ESP32 reads all sensors every 2 seconds, checks safety thresholds, triggers buzzer alarms, and sends data to the cloud via WiFi. Supervisors monitor all miners live via Blynk app on phone. SMS/email sent automatically when danger is detected.

# Chapter 2: Why ESP32?

The original reference design used an Arduino Uno paired with a separate ESP8266 chip. We upgraded to the ESP32 for the following reasons:

| Feature | Arduino Uno + ESP8266 | ESP32 (Our Choice) |
| --- | --- | --- |
| WiFi | Separate ESP8266 chip needed | Built-in WiFi ✓ |
| Bluetooth | Not available | Built-in BLE ✓ |
| CPU Speed | 16 MHz | 240 MHz (15× faster) ✓ |
| Analog Pins | 6 pins | 18 pins ✓ |
| GPIO Pins | 14 digital | 34 GPIO pins ✓ |
| Size | Two boards needed | One compact board ✓ |
| Cost | ₹300 + ₹150 = ₹450 | ~₹350 all-in-one ✓ |

The ESP32 does the job of both Arduino Uno and ESP8266 in a single compact chip. This means fewer wires, less power consumption, more sensor pins, and it fits better inside a helmet casing.

# Chapter 3: Components & Shopping List

All components listed below were selected for reliability in safety-critical environments. Do NOT substitute cheaper alternatives for gas or oxygen sensors — incorrect readings in real emergencies can be fatal.

| # | Component | Model | Purpose | Cost (₹) |
| --- | --- | --- | --- | --- |
| 1 | MCU Brain | ESP32-WROOM-32 | Main processor with WiFi + Bluetooth built-in | 400 |
| 2 | Gas / Smoke Sensor | MQ-2 | Detects LPG, Methane, Propane, Smoke | 100 |
| 3 | CO Sensor | MQ-7 | Carbon Monoxide – #1 silent killer in mines | 140 |
| 4 | Air Quality Sensor | MQ-135 | Detects NH₃, Benzene, CO₂, alcohol vapors | 100 |
| 5 | Oxygen Sensor | KE-25 / SEN0340 | Alerts when O₂ drops below 19.5% | 1200 |
| 6 | Temp + Humidity | DHT-22 | Temperature (-40 to +80°C) and humidity | 180 |
| 7 | Pressure Sensor | BMP-280 | Air pressure and underground altitude | 150 |
| 8 | Fall Detection IMU | MPU-6050 | 6-axis accelerometer + gyroscope | 100 |
| 9 | Vibration Sensor | SW-420 | Detects mine collapses or explosions | 45 |
| 10 | GPS Module | NEO-6M | Real-time miner location tracking | 450 |
| 11 | OLED Display | SSD1306 0.96" | On-helmet live sensor readout | 150 |
| 12 | Active Buzzer | 5V Piezo | Loud on-helmet alarm | 30 |
| 13 | SOS Button | Momentary (NO) | Emergency manual alert button | 20 |
| 14 | Li-Po Battery | 3.7V 2000mAh | 6-hour portable power supply | 350 |
| 15 | TP4056 Charger | With protection | Safe Li-Po charging with overcharge guard | 35 |
| 16 | Misc (wires, resistors) | Various | Breadboard, jumpers, resistors | 340 |
| ESTIMATED TOTAL |  |  |  | ₹3,690 |

PURCHASING TIP

Order from Robocraze, Robu.in, or Amazon India. Search by exact model numbers listed above. The KE-25 oxygen sensor is the costliest item — the DFRobot SEN0340 breakout board is recommended as it includes the required op-amp amplification circuit.

IMPORTANT WARNING

MQ sensors require a 24–48 hour burn-in period when first powered. Connect them to 5V for 24 hours before taking any measurements. This stabilizes the internal heating element and prevents false readings.

# Chapter 4: Simulation with Wokwi

Before building physical hardware, the entire circuit is simulated using Wokwi — a free, browser-based ESP32 simulator (wokwi.com). This allows full code testing without any risk of damaging components.

## 4.1 Steps to Simulate

- Open https://wokwi.com → New Project → Select 'ESP32'

- Add components using the '+' button: DHT22, MPU6050, BMP280, Buzzer, SSD1306, Button, LEDs

- Wire each component to the ESP32 pins as per the wiring table in Chapter 5

- Paste the ESP32 code into the Wokwi editor → Click the green Play (▶) button

- Twist the potentiometer (simulating gas sensors) past the threshold value — buzzer should activate

- Press the SOS button — verify SOS alert appears in the Serial Monitor

The Wokwi simulation link can be shared with a URL and shown to Confluence judges as documented proof of system testing prior to hardware assembly.

[ WOKWI SIMULATION SCREENSHOT — ADD IMAGE HERE ]

Insert your Wokwi simulation screenshot here showing the circuit and sensor readings.

# Chapter 5: Hardware Assembly & Pin Wiring

## 5.1 Safety Precautions

SAFETY FIRST

- Always disconnect the battery or USB before changing any wires - MQ sensors get HOT during warm-up — do not touch the metal mesh - Never short-circuit the Li-Po battery — it can catch fire - Use a multimeter to verify 3.3V and 5V rails before connecting sensors

## 5.2 Pin Wiring Reference Table

| Sensor / Component | ESP32 Pin | Protocol | Notes |
| --- | --- | --- | --- |
| DHT-22 (Temp/Humidity) | GPIO 4 | 1-Wire | Add 10kΩ pull-up between DATA and 3.3V |
| MQ-2 (Gas/Smoke) | GPIO 34 (ADC) | Analog | Voltage divider needed – 5V→2.5V for ESP32 ADC |
| MQ-7 (CO Sensor) | GPIO 32 (ADC) | Analog | Voltage divider needed; 5V heater |
| MQ-135 (Air Quality) | GPIO 33 (ADC) | Analog | Voltage divider needed; 5V heater |
| KE-25 (Oxygen) | GPIO 36 (ADC) | Analog | LM358 op-amp amplifier or DFRobot breakout |
| BMP-280 (Pressure) | GPIO 21/22 | I2C | SDA=21, SCL=22; Address 0x76 or 0x77 |
| MPU-6050 (Fall) | GPIO 21/22 + 19 | I2C | Shared I2C bus; INT pin on GPIO 19 |
| OLED SSD1306 | GPIO 21/22 | I2C | Shared I2C bus; Address 0x3C |
| SW-420 (Vibration) | GPIO 18 | Digital | Adjust sensitivity with onboard potentiometer |
| NEO-6M GPS | GPIO 16 (RX2) / 17 (TX2) | UART2 | GPS TX → ESP32 RX2; baud 9600 |
| Active Buzzer | GPIO 26 | Digital OUT | HIGH = alarm ON; active buzzer (no freq needed) |
| SOS Button | GPIO 5 | Digital IN | INPUT_PULLUP; press pulls to GND |

CRITICAL: VOLTAGE DIVIDER FOR MQ SENSORS

ESP32 ADC pins accept a maximum of 3.3V. MQ gas sensors output 0–5V. Without a voltage divider, you will permanently damage the ESP32 chip. Circuit: MQ AOUT → 10kΩ resistor → ESP32 ADC Pin → 10kΩ resistor → GND

## 5.3 Assembly Phases

Phase 1: Mount ESP32 on Breadboard

- Place ESP32 in the center of the breadboard straddling the center groove

- Connect 3.3V pin → positive rail (red), GND pin → negative rail (black)

- USB port should face the breadboard edge for easy access

Phase 2: I2C Sensors (BMP280, MPU-6050, OLED)

- GPIO 21 = SDA bus, GPIO 22 = SCL bus — all three I2C devices share these two wires

- Add 4.7kΩ pull-up resistors between SDA and 3.3V, and between SCL and 3.3V

- I2C addresses: BMP280 = 0x76, MPU-6050 = 0x68, OLED = 0x3C

Phase 3: Gas Sensors (MQ-2, MQ-7, MQ-135)

- All MQ sensor VCC pins connect to 5V (Vin) rail — NOT 3.3V

- Install voltage dividers on all AOUT lines (10kΩ + 10kΩ) before connecting to ESP32

Phase 4: GPS Module (NEO-6M)

- GPS TX → ESP32 GPIO 16 (RX2), GPS RX → ESP32 GPIO 17 (TX2)

- Attach the ceramic patch antenna. GPS needs 30–60 seconds to acquire satellite fix

[ HARDWARE ASSEMBLY PHOTO — ADD IMAGE HERE ]

Insert your breadboard assembly photograph here.

# Chapter 6: Software & Arduino Code

## 6.1 Libraries Required

| Library Name | Search Term | Purpose |
| --- | --- | --- |
| DHT sensor library | DHT sensor library | Temperature & humidity (DHT-22) |
| Adafruit BMP280 | BMP280 | Barometric pressure sensor |
| MPU6050 by Electronic Cats | MPU6050 | Fall detection accelerometer |
| Adafruit SSD1306 | SSD1306 | OLED display driver |
| Adafruit GFX Library | Adafruit GFX | Required graphics base for OLED |
| TinyGPS++ | TinyGPSPlus | GPS NMEA data parser |
| BlynkSimpleEsp32 | Blynk | IoT cloud dashboard |

## 6.2 Key Code Logic

The ESP32 code runs a continuous loop that executes every 2 seconds:

- readAllSensors() — reads data from all 10 sensors simultaneously

- checkThresholds() — compares readings against safe limits. Triggers buzzer if exceeded

- updateOLED() — refreshes the on-helmet OLED display with current values

- sendToBlynk() — pushes all sensor data to the cloud dashboard via WiFi

- readGPS() — parses NMEA data from the NEO-6M GPS module

## 6.3 Alarm Threshold Values

| Parameter | Threshold | Action Triggered |
| --- | --- | --- |
| CO Level (MQ-7) | > 300 ADC | Buzzer + Blynk alert + SMS |
| Gas / LPG (MQ-2) | > 400 ADC | Buzzer + Blynk alert + SMS |
| Air Quality (MQ-135) | > 400 ADC | Warning notification |
| Oxygen Level (KE-25) | < 850 ADC (~19.5%) | Buzzer + critical alert |
| Temperature | > 40°C | Warning + dashboard alert |
| Fall Detected (MPU-6050) | G-force > 2.5G or < 0.3G | Fall alert + Blynk event |
| Vibration (SW-420) | Digital HIGH | Collapse warning + alert |
| SOS Button | Button pressed | Immediate SMS + email + alert |

# Chapter 7: IoT Dashboard — Blynk Setup

Blynk is a free IoT platform that turns your smartphone into a live dashboard. Supervisors can monitor all miners from a single screen and receive instant push notifications when any danger threshold is exceeded.

## 7.1 Setup Steps

- Download 'Blynk IoT' app on Android or iPhone

- Register at blynk.cloud → New Template → Name: 'Smart Helmet' → Hardware: ESP32 → Connection: WiFi

- Create Datastreams for each virtual pin (see table below)

- Build dashboard: add Gauge, Chart, Value Display, Map, and LED widgets

- Create Events: co_alert, gas_alert, o2_alert, fall_alert, sos, vibration_alert → Enable push + email notifications

- Copy Auth Token from Device Info → paste into ESP32 code as auth[]

## 7.2 Datastream Configuration

| Virtual Pin | Name | Data Type | Range | Units |
| --- | --- | --- | --- | --- |
| V0 | Temperature | Double | -40 to 80 | °C |
| V1 | Humidity | Double | 0 to 100 | % RH |
| V2 | CO Level (MQ-7) | Integer | 0 to 4095 | ADC |
| V3 | Gas / LPG (MQ-2) | Integer | 0 to 4095 | ADC |
| V4 | Air Quality (MQ-135) | Integer | 0 to 4095 | ADC |
| V5 | Oxygen % | Double | 0 to 25 | % O₂ |
| V6 | Pressure | Double | 300 to 1100 | hPa |
| V7 | Altitude | Double | -500 to 9000 | meters |
| V10 | GPS Latitude | Double | -90 to 90 | degrees |
| V11 | GPS Longitude | Double | -180 to 180 | degrees |

[ BLYNK DASHBOARD SCREENSHOT — ADD IMAGE HERE ]

Insert screenshot of your Blynk mobile/web dashboard showing live sensor gauges and GPS map.

# Chapter 8: Testing & Troubleshooting

## 8.1 Sensor Test Procedures

DHT-22 Temperature & Humidity

- Open Serial Monitor at 115200 baud — should show ~25°C and ~50% humidity

- Hold sensor in your fist for 30 seconds → temperature should rise

- Blow on it gently → humidity should increase

Gas Sensors (MQ-2, MQ-7, MQ-135)

- Allow 5 minutes warm-up after powering on before trusting readings

- Hold a lighter (without flame, just gas release) near MQ-2 → ADC value should spike

- For classroom testing: breathe gently on sensor — exhaled CO₂ raises MQ-135 reading

Fall Detection (MPU-6050)

- Hold breadboard level → G-force reading should be ~1.0G

- Tilt rapidly to 90 degrees → acceleration values change and fall alert triggers

GPS Module (NEO-6M)

- Take setup near a window or outdoors → wait 1–2 minutes for satellite fix

- Blue LED on NEO-6M blinks at 1Hz when fix is acquired

- Copy coordinates from Serial Monitor → verify in Google Maps

## 8.2 Troubleshooting Guide

| Problem | Likely Cause | Solution |
| --- | --- | --- |
| Upload fails – 'Connecting...' | ESP32 not in download mode | Hold BOOT button during upload |
| DHT-22 shows nan or -999 | Pull-up resistor missing | Add 10kΩ between DATA and 3.3V |
| Gas sensor always 4095 | Voltage divider missing | Add 10kΩ + 10kΩ divider on AOUT |
| BMP280 / MPU not found | Wrong I2C address | Try 0x77 for BMP, 0x69 for MPU |
| GPS never gets fix | Indoors / no antenna | Move near window, wait 2 minutes |
| OLED blank | Wrong I2C address or no power | Check 3.3V supply, try 0x3D |
| Blynk disconnects often | Weak WiFi signal | Move closer to router, check SSID/pass |
| Buzzer always ON | Threshold too low / sensor cold | Increase threshold values; let sensors warm up |

# Chapter 9: Conclusion & Future Scope

## 9.1 Project Summary

The IoT Smart Safety Helmet successfully transforms a passive piece of protective equipment into an active, AI-assisted life-saving system. For a total component cost of approximately ₹3,690, a coal miner now gains:

- Continuous monitoring of 6 environmental hazards (CO, gas, air quality, oxygen, temperature, pressure)

- Real-time GPS location visible to supervisors on a live dashboard

- Instant fall and vibration detection with automatic alert dispatch

- On-helmet OLED display showing live readings without needing a phone

- Cloud connectivity via WiFi sending push notifications and SMS to supervisors

- 6-hour portable battery operation independent of external power

## 9.2 Future Scope

- Machine Learning integration to predict dangerous gas buildup patterns before thresholds are crossed

- LoRa wireless communication for deep underground areas where WiFi signals cannot reach

- Custom PCB design to replace the breadboard prototype for a production-ready wearable helmet

- Multi-miner dashboard showing all team members on a single supervisor screen with real-time map

- Voice alert system added to the helmet for audible status updates in noisy mine environments

- Integration with mine ventilation control systems — helmet data triggers fans automatically

This project demonstrates that technology can save lives — and for under ₹4,000, it can.

## 9.3 Team

| Group Members Sonvi Aayush Anand Ananya Suman | Institution Bharati Vidyapeeth College of Engineering Confluence 2026 |
| --- | --- |